General Loss Report

Date of Loss:

9/28/2024

Insurable Interest:

Richard Daly

392 HEATH STBAXLEY, GA31513

Dwelling Description:

The insured dwelling is a single-story residential structure with brick veneer exterior elevations. The roofing system consists of three-tab asphalt shingles with a rated life expectancy of approximately 25 years.

Property Condition:

No observable hazards or adverse conditions were noted at the property that would contribute to an increased risk exposure.

Inspection:

An on-site inspection was completed on November 13, 2024, at the insured location with Richard Daly present. Findings from the inspection are outlined in the sections below.

Dwelling

Roof - The dwelling is covered with a single-layer, three-tab shingle roof system with an approximate pitch of 5/12. The member advised the roof age to be approximately 20 years. The loss was reported as wind damage related to Hurricane Helene.

Our inspection revealed wind damage to the roof covering of the dwelling.

Front Slope- 13 wind-damaged shingles identified

Right Slope- No storm-related damage observed

Rear Slope- Approximately 150 wind-damaged shingles noted

Left Slope- No storm damage observed

All remaining roof areas were inspected and did not exhibit storm-related damage.

Due to the extent of damage affecting more than 70% of the roof surface area, along with the age and condition of the shingles, full roof replacement is recommended. Applicable line items have been included. Refer to the sketch and photo documentation for supporting details.

Elevations

Front elevation- Fascia observed displaced

Right elevation- No storm-related damage noted

Rear elevation- Missing fascia and displaced soffit observed

Left elevation- Fascia displaced

All remaining exterior components, including siding, soffit, fascia, windows, doors, gutters, and downspouts, were inspected and found free of storm-related damage.

Interior

Member has stated that no storm related interior damage has been noted at the time of this report.

Other Structures

Detached Garage- Detached garage roof is comprised of 3-tab shingles, 25 year 3-tab shingles.

Right slope- Right slope is free from wind damage

Left slope- Left slope has 26 wind damaged shingles

Elevations

Front elevation-  Front elevation is free from wind damage

Right elevation- Right elevation is free from wind damage

Rear elevation- Rear elevation is free from wind damage

Left elevation- Left elevation is free from wind damage

Fencing: The insured privacy fence consists of six-foot PVC panels. All fencing runs are owned by the member and are located along the front, right, and left sides of the property..

Fencing has been damaged during the storm and needs to be repaired as follows:

Front Run 5 panels missing from front run of fence, One gate is missing from storm

Right Run: 10 panels missing and damaged from storm

Rear Run: No damage observed

Left Run: 5 panels missing from left run

Necessary fencing repair line items have been added. Refer to the photo report for additional documentation.

Contents

The member reported a power outage on the date of loss, resulting in food spoilage with an estimated value of $500.

Pool- The member reported that fence panels were blown into the swimming pool during the storm, causing damage. Additionally, the following items were reported as damaged due to the storm and supported by photos provided by the member:

- Wheel Barrow

- Car Top Storage

- Flag Pole

Review

The scope of damages was reviewed with Richard Daly. The member was advised that coverage determinations and payment decisions are made by USAA. Recoverable depreciation was discussed, including the process for reimbursement following completed repairs. The member acknowledged understanding and did not express additional concerns.

The member confirmed access to their USAA online account and was advised that estimates would be available upon completion of review.

Supplement

N/A

Priors:

No prior losses for this address

Code Items

N/A

Overhead & Profit

Overhead and profit were not included, as the involvement of a general contractor is not anticipated.

MICA/QA Assist

MICA/QA Assist was utilized. Recommendations related to fencing removal and replacement were reviewed; however, removal labor was not applied, as fencing components were displaced by wind rather than requiring manual removal.

Mortgagee Information

Mortgage information was reviewed and confirmed. The mortgage company on record is PennyMac.

Cause and Origin:

The loss is attributed to wind damage with an approximate date of loss of 09/28/2024. This assignment was received on 11/08/2024.

Subrogation:

No potential of subrogation found, as this loss is weather related.

Salvage:

No salvage opportunities noted.